## SIMATS ENGINEERING

## Assignment Question

## Name: VARSHITHA MAHADEVAN

Reg No.:192511147

## COURSE NAME: Artificial Intelligence

## COURSE CODE: CSA1715

## COURSE OUTCOME COVERED

## CO4Implement machine learning techniques including decision tree algorithms,

## reinforcement learning, and build models for classification and decision-making.

## CO5Design AI-based systems using PySpark, RDD operations, and intelligent agent

## architectures, using scalable systems and integrate components in distributed AI

## applications.

## Assessment Tool Weightage:

| ## QUESTIONS Total Marks: 100 |  |
| --- | --- |
| ## Q.No ## 1 | ## Question Blooms Level Marks ## BL4,BL5 100 |

## 1. PROBLEM STATEMENT

Traditional irrigation systems usually supply water at fixed time intervals without considering the actual moisture condition of the soil. This can result in over-irrigation, under-irrigation, water wastage, and poor crop growth.

The AI-Based Smart Irrigation System is proposed to automatically monitor soil conditions and decide when irrigation is required. The system uses soil moisture and environmental parameters as input and applies an Artificial Intelligence/Machine Learning model to determine whether the water pump should be turned ON or OFF.

The main aim is to reduce unnecessary water usage while maintaining suitable soil moisture for healthy plant growth.

# 2. OBJECTIVE

The main objectives of the project are:

- To develop an intelligent irrigation system using Artificial Intelligence.

- To monitor soil moisture automatically.

- To predict whether irrigation is required based on sensor values.

- To automatically control the water pump.

- To reduce water wastage.

- To minimize manual monitoring of agricultural fields.

- To improve irrigation efficiency and support healthy crop growth.

- To demonstrate the application of Machine Learning in smart agriculture.

# 3. REQUIREMENTS AND ENVIRONMENT USED

## 3.1 Hardware Requirements

- Soil moisture sensor

- Temperature sensor

- Humidity sensor

- Arduino/ESP32 microcontroller

- Water pump

- Relay module

- Water tank

- Connecting wires

- Breadboard

- Power supply

- Plants/soil for testing

## 3.2 Software Requirements

- Python

- Jupyter Notebook / Google Colab

- Arduino IDE

- Python libraries:

  - NumPy

  - Pandas

  - Scikit-learn

  - Matplotlib

## 3.3 Machine Learning Model

A Decision Tree Classifier can be used for predicting irrigation requirements.

### Input Parameters

- Soil moisture

- Temperature

- Humidity

### Output

- 1 → Irrigation Required

- 0 → Irrigation Not Required

# 4. DESIGN / PROPOSED SOLUTION

The proposed system consists of four major stages:

### Stage 1 – Data Collection

Sensors collect information about the soil and surrounding environment.

### Stage 2 – Data Processing

The collected sensor values are sent to the microcontroller and processed before being given to the AI model.

### Stage 3 – AI Prediction

The Machine Learning model analyzes the input values and predicts whether irrigation is required.

### Stage 4 – Automatic Irrigation

Based on the prediction, the controller activates or deactivates the water pump through a relay.

## System Architecture

Sensors → Microcontroller → Data Processing → AI Model → Decision → Relay → Water Pump

When the soil is sufficiently dry, the AI model predicts that irrigation is required and the pump is activated.

When sufficient moisture is detected, the pump remains OFF.

## Advantages

- Saves water

- Reduces manual work

- Provides automatic irrigation

- Uses real-time sensor information

- Can be extended to IoT-based agriculture

- Supports efficient crop management

# 5. ALGORITHM / PSEUDOCODE / FLOWCHART

## 5.1 Algorithm

- Start the system.

- Read soil moisture value.

- Read temperature value.

- Read humidity value.

- Preprocess the sensor data.

- Provide the values to the trained AI model.

- Predict irrigation requirement.

- If prediction = 1:

  - Turn ON the water pump.

- Otherwise:

  - Keep the water pump OFF.

- Continue monitoring.

- Stop.

## 5.2 Pseudocode

START

Initialize sensors

Initialize microcontroller

Initialize water pump and relay

Load trained AI model

WHILE system is running:

Read soil moisture

Read temperature

Read humidity

Input = [soil moisture, temperature, humidity]

Prediction = AI_Model(Input)

IF Prediction == 1 THEN

Turn ON water pump

Display "Irrigation Required"

ELSE

Turn OFF water pump

Display "Irrigation Not Required"

END IF

END WHILE

STOP

## 5.3 Flowchart

START

↓

Initialize Sensors

↓

Read Soil Moisture,

Temperature & Humidity

↓

Process Data

↓

AI/ML Prediction

↓

┌───────────────┐

│ Irrigation    │

│ Required?     │

└───────┬───────┘

YES │ NO

↓   ↓

Pump ON  Pump OFF

↓   ↓

Display Result

↓

Continue Monitoring

↓

STOP

# 6. IMPLEMENTATION / SOURCE CODE

## 6.1 AI Model – Python

from sklearn.tree import DecisionTreeClassifier

# Training data

X = [

[20, 35, 80],

[25, 30, 75],

[30, 28, 70],

[35, 25, 60],

[40, 22, 55],

[45, 20, 50],

[50, 18, 45],

[55, 17, 40]

]

# 1 = Irrigation Required

# 0 = Irrigation Not Required

Y = [1, 1, 1, 1, 0, 0, 0, 0]

# Create and train model

model = DecisionTreeClassifier()

model.fit(X, Y)

# Test input

soil_moisture = 25

temperature = 30

humidity = 75

prediction = model.predict(

[[soil_moisture, temperature, humidity]]

)

if prediction[0] == 1:

print("Irrigation Required")

else:

print("Irrigation Not Required")

### Sample Output

Irrigation Required

## 6.2 Arduino/ESP32 Control Logic

int moistureSensor = A0;

int relay = 7;

void setup()

{

pinMode(moistureSensor, INPUT);

pinMode(relay, OUTPUT);

Serial.begin(9600);

}

void loop()

{

int moisture = analogRead(moistureSensor);

Serial.print("Soil Moisture: ");

Serial.println(moisture);

if (moisture < 400)

{

digitalWrite(relay, HIGH);

Serial.println("Irrigation Required - Pump ON");

}

else

{

digitalWrite(relay, LOW);

Serial.println("Irrigation Not Required - Pump OFF");

}

delay(2000);

}

Note: The threshold value should be calibrated according to the actual sensor and soil conditions. In a complete AI implementation, the ML prediction can be used instead of a fixed threshold.

# 7. TEST CASES AND EXPECTED / ACTUAL RESULTS

| Test Case | Soil Moisture | Temperature | Humidity | Expected Result | Actual Result |
| --- | --- | --- | --- | --- | --- |
| TC01 | 20 | 35°C | 80% | Irrigation Required | Irrigation Required |
| TC02 | 25 | 30°C | 75% | Irrigation Required | Irrigation Required |
| TC03 | 35 | 25°C | 60% | Irrigation Required | Irrigation Required |
| TC04 | 45 | 22°C | 55% | Irrigation Not Required | Irrigation Not Required |
| TC05 | 50 | 20°C | 50% | Irrigation Not Required | Irrigation Not Required |
| TC06 | 55 | 17°C | 40% | Irrigation Not Required | Irrigation Not Required |

### Test Result

The system successfully identifies dry and sufficiently moist conditions and provides an appropriate irrigation decision.

# 8. EXECUTION SCREENSHOTS / OUTPUT

The following screenshots can be included in the final report:

### Screenshot 1 – Python Program

Show the Python code and execution result:

Irrigation Required

### Screenshot 2 – AI Model Prediction

Show the input values:

Soil Moisture = 25

Temperature = 30°C

Humidity = 75%

Output:

Irrigation Required

### Screenshot 3 – Pump ON

Soil Moisture: 25

Irrigation Required - Pump ON

### Screenshot 4 – Pump OFF

Soil Moisture: 50

Irrigation Not Required - Pump OFF

The actual screenshots should be captured while executing the program and hardware prototype.

# 9. ANALYSIS AND DISCUSSION

The AI-Based Smart Irrigation System provides an automatic method for determining irrigation requirements.

The soil moisture sensor provides information about the current soil condition. Temperature and humidity provide additional environmental information. These parameters are given to the Machine Learning model for prediction.

A Decision Tree Classifier is suitable for this project because it is simple to understand and can make decisions based on different input conditions.

When the soil is dry, the model predicts Irrigation Required, and the controller activates the pump. When sufficient moisture is present, the model predicts Irrigation Not Required, preventing unnecessary watering.

The system can therefore reduce manual intervention and improve the efficiency of water usage.

### Limitations

- Sensor readings can be affected by soil type.

- The model requires suitable training data.

- Sensor calibration is necessary.

- Hardware implementation requires a reliable power supply.

- Weather conditions may affect irrigation requirements.

### Future Enhancement

The system can be improved by:

- Adding IoT connectivity.

- Developing a mobile application.

- Using real-time weather information.

- Adding multiple soil moisture sensors.

- Using larger agricultural datasets.

- Implementing advanced ML models.

- Providing cloud-based monitoring.

- Sending alerts to farmers.

# 10. CONCLUSION

The AI-Based Smart Irrigation System successfully demonstrates the use of Artificial Intelligence and Machine Learning in smart agriculture.

The system collects soil and environmental information and uses an AI model to determine whether irrigation is required. Based on the prediction, the water pump can be automatically controlled.

The proposed system can help reduce water wastage, minimize manual monitoring, and improve irrigation efficiency. With IoT connectivity, larger datasets, weather prediction, and advanced Machine Learning algorithms, the system can be further developed into a practical smart farming solution.

11. Sustainable Development Goals (SDG)

SDG 2 – Zero Hunger
The project supports sustainable agriculture by providing water to crops based on their actual irrigation requirements, which can help improve crop productivity.

SDG 6 – Clean Water and Sanitation
The system helps reduce unnecessary water usage by automatically controlling irrigation according to soil moisture conditions, supporting efficient water management.

SDG 7 – Affordable and Clean Energy
The automated pump operates only when irrigation is required, which can help avoid unnecessary energy consumption.

SDG 12 – Responsible Consumption and Production
The system promotes responsible use of water and energy resources through automated and optimized irrigation.

SDG 13 – Climate Action
Efficient use of water and energy can contribute to more sustainable agricultural practices and help reduce resource wastage.

# 12. INDIVIDUAL CONTRIBUTION OF GROUP MEMBERS

| Group Member | Contribution |
| --- | --- |
| Member 1 | Problem identification, project planning, and documentation |
| Member 2 | Sensor selection, circuit design, hardware setup, and AI model development |
| Member 3 | Python programming, hardware-software integration, testing, result analysis, screenshots, and presentation |

# 12. REFERENCES

- Stuart Russell and Peter Norvig, Artificial Intelligence: A Modern Approach, Pearson.

- Scikit-learn Documentation, Decision Tree Classifier.

- Python Documentation, Python Programming Language.

- Arduino Documentation, Arduino Programming and Hardware.

- NumPy Documentation, Numerical Computing with Python.

- Pandas Documentation, Data Analysis with Python.

- Research articles related to Artificial Intelligence and Smart Irrigation Systems.

- Research articles related to Machine Learning applications in precision agriculture.

# PROJECT SUMMARY

Project Title: AI-Based Smart Irrigation System

Domain: Artificial Intelligence / Machine Learning / Smart Agriculture

Input: Soil Moisture, Temperature, Humidity

AI Algorithm: Decision Tree Classification

Output: Irrigation Required / Irrigation Not Required

Hardware: Soil Moisture Sensor, Temperature/Humidity Sensor, Microcontroller, Relay and Water Pump

Main Benefit: Automatic irrigation and reduction of unnecessary water usage.